# SafeSense

IoT-Based Rain and Flood Hazard Monitoring
with Multi-Channel Smart Emergency Alert System

Arduino Uno  •  ESP32-CAM  •  SIM900A GSM  •  WiFi  •  Camera Capture  •  PHP Dashboard

# What is IoT?

IoT stands for Internet of Things. It refers to the network of physical devices — such as sensors, microcontrollers, and everyday objects — that are connected to the internet and can collect, send, and receive data without human intervention.

Think of it this way: a regular Arduino system can only work locally — it reacts to sensors and triggers outputs like LEDs or a buzzer. But an IoT-enabled Arduino can also send that data over the internet in real time, making it visible on a web dashboard, triggering messages to other devices, or alerting remote stations automatically.

| Without IoT (Original) | With IoT (Updated SafeSense) |
| --- | --- |
| Works locally only | Connected to the internet via ESP32-CAM built-in WiFi |
| Sends SMS alerts only | Sends SMS + internet alerts + camera images |
| No internet, no remote dashboard | Real-time web dashboard accessible from any device |
| No direct link to stations | Auto-notifies Medical & Police Stations via HTTP POST |
| No visual evidence | Camera captures flood/accident images automatically |

# WiFi Connectivity — No Separate Shield Needed

The ESP32-CAM module has built-in WiFi capability. Unlike a basic Arduino Uno, the ESP32-CAM can connect directly to any standard WiFi network and communicate over the internet using HTTP protocols. This eliminates the need for a separate WiFi shield or module.

The ESP32-CAM acts as the WiFi gateway in the SafeSense system. It receives sensor data from the Arduino Uno via a Serial (UART) connection, then transmits that data over WiFi to the hospital management web dashboard using HTTP POST requests with JSON payloads.

Important: You do NOT need to purchase a separate WiFi shield. The ESP32-CAM IS the WiFi module. It has a built-in 2.4GHz WiFi radio that handles all internet communication.

# Objectives

To detect rain using a rain sensor module

To measure water levels using a resistive water level sensor (analog)

To activate LED hazard lights (Green/Yellow/Red) when dangerous conditions are detected

To detect possible accidents using a vibration sensor with multi-detection confirmation

To send SMS alerts using the SIM900A GSM module when a critical event occurs

To connect the system to the internet using the ESP32-CAM's built-in WiFi (no separate shield needed)

To transmit real-time sensor data and alerts to the Hospital Management web dashboard via HTTP POST

To automatically capture camera images (JPEG) during DANGER and CRITICAL events as visual evidence

To upload captured images to the web server for viewing by hospital staff and emergency responders

To automatically notify the nearest Medical Station and Police Station through internet-based messaging

To send periodic heartbeat pings so the dashboard knows if the field device is online

To reduce road accidents during rainy and flood conditions

To improve emergency response time through multi-channel alerting (SMS + WiFi + Camera)

To provide a low-cost, IoT-enabled, and deployable solution for local communities

# System Architecture

SafeSense uses a dual-microcontroller architecture where each board handles what it does best:

| Component | Board | Role |
| --- | --- | --- |
| Sensor Controller | Arduino Uno (ATmega328P) | Reads all sensors, controls LEDs, sends SMS via SIM900A GSM, forwards data to ESP32-CAM via Serial |
| WiFi Gateway + Camera | AI-Thinker ESP32-CAM | Receives data from Arduino via Serial, connects to WiFi, POSTs JSON alerts to dashboard, captures and uploads JPEG images |

## Why Two Microcontrollers?

| Concern | Arduino Uno | ESP32-CAM |
| --- | --- | --- |
| Analog sensors | ✅ Reliable 10-bit ADC | ⚠️ ADC2 conflicts with WiFi |
| Real-time I/O | ✅ Deterministic timing | ⚠️ WiFi stack can cause jitter |
| WiFi | ❌ No built-in WiFi | ✅ Built-in 2.4GHz WiFi |
| Camera | ❌ No camera support | ✅ Built-in OV2640 camera |
| GSM (SoftwareSerial) | ✅ Works well | ⚠️ No SoftwareSerial on ESP32 |
| Memory | 2 KB SRAM (tight) | 520 KB SRAM + 4MB PSRAM |

## Communication Flow

Arduino Uno reads sensors every 2 seconds → evaluates alert level → if alert triggered, sends structured data packet over Serial to ESP32-CAM → ESP32-CAM parses the data → POSTs JSON alert to web server → if DANGER or CRITICAL, also captures a camera image and uploads it → web dashboard displays the alert with image to hospital staff within 5 seconds.

# Features

## Sensor Features

Rain Detection: Detects presence of rain using digital output. The rain sensor module with LM393 comparator outputs LOW when rain is detected.

Water Level Monitoring: Resistive water level sensor on analog pin A0 measures water depth proportionally. Readings are averaged over 5 samples for stability.

Accident Detection: SW-420 vibration sensor detects sudden impacts. Requires 3 confirmed detections within 10 seconds to avoid false positives from wind or passing vehicles.

Adaptive Hazard Lighting: 3 LEDs (Green/Yellow/Red) with non-blocking blink patterns: solid green = safe, solid yellow = warning, slow red blink = danger, fast red+yellow alternating = critical.

Audible Buzzer (Optional): Piezo buzzer on Arduino pin D9. Emits slow 1kHz beeps during DANGER events and rapid 2kHz beeps during CRITICAL events. Disabled by default in software — enable by setting BUZZER_ENABLED = true after wiring.

## Communication Features

SMS Alerts (SIM900A GSM): Sends automated SMS text messages to predefined emergency phone numbers when DANGER or CRITICAL events occur. Uses AT commands via SoftwareSerial on Arduino pins D7/D8. Separate 5-minute cooldown to control costs.

WiFi HTTP Alerts (ESP32-CAM): POSTs JSON payloads to the hospital web dashboard over WiFi. Includes retry with exponential backoff (2s, 4s, 8s). Auto-reconnects WiFi if connection drops.

Camera Image Capture: The ESP32-CAM's built-in OV2640 camera captures JPEG images (800×600) during DANGER and CRITICAL alerts. Uses the onboard flash LED for illumination in dark/rain conditions. Images are uploaded to the server as visual evidence.

Heartbeat Pings: ESP32-CAM sends a keep-alive HTTP POST every 5 minutes with device status (WiFi signal strength, uptime, free memory, camera status). Dashboard knows if the device is online.

Dual-Channel Alerting: The system uses both GSM (SMS) and WiFi (Internet) simultaneously. If one channel is unavailable, the other serves as backup.

## Safety Features

Hardware Watchdog Timer: Both Arduino Uno (8-second WDT) and ESP32-CAM (10-second WDT) have hardware watchdog timers that automatically reset the board if the main loop hangs.

Non-Blocking Architecture: All timing uses millis() instead of delay(), ensuring sensors are read continuously even during LED blink patterns.

Vibration Confirmation Logic: Requires multiple vibration detections (default: 3) within a time window (default: 10 seconds) before triggering an accident alert, reducing false positives.

Alert Cooldown: 60-second cooldown between WiFi alerts and 5-minute cooldown between SMS to prevent spam.

Power Stability: Separate voltage regulators: 5V for Arduino+ESP32 and ~4V for SIM900A. Prevents brown-out resets during GSM transmission bursts.

# Hardware Components — Bill of Materials

| # | Component | Qty | Purpose |
| --- | --- | --- | --- |
| 1 | Arduino Uno R3 | 1 | Sensor controller + GSM SMS |
| 2 | ESP32-CAM (AI-Thinker) | 1 | WiFi gateway + camera (built-in WiFi & OV2640 camera) |
| 3 | SIM900A GSM Module | 1 | SMS alerts (requires 4V supply, SIM card) |
| 4 | Resistive Water Level Sensor | 1 | Measures water depth (analog output) |
| 5 | Rain Sensor Module (LM393) | 1 | Detects rain (digital output, LOW = rain) |
| 6 | SW-420 Vibration Sensor | 1 | Detects impact/accident (digital output) |
| 7 | LEDs (5mm, Green/Yellow/Red) | 3 | Visual hazard status indicators |
| 8 | 220Ω Resistors | 3 | LED current limiting (one per LED) |
| 9 | Piezo Buzzer (optional) | 1 | Audible alarm during DANGER/CRITICAL (wire to D9) |
| 10 | LM2596 Buck Converter | 1-2 | Power regulation (5V + 4V outputs) |
| 11 | SIM Card (any carrier) | 1 | For GSM SMS (needs SMS plan & credit) |
| 12 | Jumper Wires | ~30 | Connections |
| 13 | Breadboard | 1-2 | Prototyping |
| 14 | Battery Pack (7-12V) | 1 | Power source |
| 15 | FTDI USB-to-Serial Adapter | 1 | ESP32-CAM programming (no USB on board) |

# Pin Connection Details

## Arduino Uno Connections

| Arduino Pin | Direction | Connected To |
| --- | --- | --- |
| A0 | INPUT | Water Level Sensor — Signal |
| D2 | INPUT | Vibration Sensor — OUT |
| D3 | INPUT | Rain Sensor — DO |
| D7 | INPUT (SoftSerial RX) | SIM900A — TX |
| D8 | OUTPUT (SoftSerial TX) | SIM900A — RX |
| D9 | OUTPUT (optional) | Buzzer — + pin |
| D10 | OUTPUT | LED Green — Anode (via 220Ω to GND) |
| D11 | OUTPUT | LED Yellow — Anode (via 220Ω to GND) |
| D12 | OUTPUT | LED Red — Anode (via 220Ω to GND) |
| TX (D1) | OUTPUT | ESP32-CAM — U0R (GPIO3/RX) |
| RX (D0) | INPUT | ESP32-CAM — U0T (GPIO1/TX) |
| 5V | POWER | Sensor VCCs |
| GND | GROUND | All sensor GNDs, LED cathodes |

## ESP32-CAM Connections

| ESP32-CAM Pin | Direction | Connected To |
| --- | --- | --- |
| U0R (GPIO3) | INPUT | Arduino Uno — TX (D1) |
| U0T (GPIO1) | OUTPUT | Arduino Uno — RX (D0) |
| 5V / VCC | POWER | 5V from LM2596 (board has onboard 3.3V regulator) |
| GND | GROUND | Common GND |
| GPIO33 | OUTPUT | Onboard status LED (active LOW, built-in) |
| GPIO4 | OUTPUT | Onboard flash LED / camera flash (active HIGH, built-in) |
| Camera | — | OV2640 camera module (built into board, no wiring needed) |

## SIM900A GSM Connections

| SIM900A Pin | Connected To | Notes |
| --- | --- | --- |
| VCC | 4V from LM2596 | CRITICAL: Max 4.2V! Never connect to 5V! |
| GND | Common GND | Shared with Arduino + ESP32 |
| TX | Arduino D7 | SIM900A sends data to Arduino |
| RX | Arduino D8 | Arduino sends AT commands to SIM900A |

# Power Distribution

⚠️ VERY IMPORTANT — Do NOT skip this section or your system will reset randomly.

Use a Battery connected to an LM2596 Buck Converter module for power regulation:

Battery (7-12V) → LM2596 Buck Converter #1 → Output 5V → Arduino Uno VIN + ESP32-CAM VCC + Sensor VCCs

Battery (7-12V) → LM2596 Buck Converter #2 → Output ~4V → SIM900A VCC (SEPARATE regulator!)

ALL GROUNDS must be connected together (Arduino GND + ESP32 GND + SIM900A GND + Sensor GNDs)

The SIM900A draws up to 2A peak during SMS transmission — it MUST have its own power regulator

Measure output voltages with a multimeter BEFORE connecting any components

# Alert Levels

| Level | Trigger Condition | LED Pattern | Buzzer | SMS | WiFi POST | Camera |
| --- | --- | --- | --- | --- | --- | --- |
| 🟢 Safe | No rain, low water, no vibration | Solid green | — | — | — | — |
| 🟡 Warning | Rain detected OR water rising | Solid yellow | — | — | ✅ | — |
| 🟠 Danger | High water + rain | Yellow + red slow blink | Slow beep | ✅ | ✅ | ✅ Capture + Upload |
| 🔴 Critical | Flood threshold OR confirmed accident | Red + yellow fast alternating | Rapid beep | ✅ | ✅ | ✅ Capture + Upload |

# Software — Arduino Sketch Files

| File | Upload To | Purpose |
| --- | --- | --- |
| SafeSense_Arduino.ino | Arduino Uno | Sensors, LEDs, buzzer, GSM SMS, Serial bridge to ESP32 |
| SafeSense_ESP32CAM.ino | ESP32-CAM | WiFi HTTP POST, camera capture & upload, heartbeat |
| SafeSense_ESP32_Standalone.ino | ESP32 DevKit (alternative) | All-in-one single board version |
| SafeSense_IoT.ino | ⚠️ DEPRECATED | Original single-board sketch — for reference only |

## Required Libraries

ArduinoJson by Benoit Blanchon (v6.x or v7.x) — install via Arduino Library Manager

SoftwareSerial — built-in with Arduino IDE (for Arduino Uno)

WiFi.h — built-in with ESP32 board package

HTTPClient.h — built-in with ESP32 board package

esp_camera.h — built-in with ESP32 board package

# Web Dashboard (Hospital Management System)

The SafeSense Hospital Management System is a PHP/MySQL web application that receives IoT alerts from the Arduino device and displays them to hospital staff in real time. It also manages day-to-day hospital operations (patients, doctors, appointments, billing, EMR).

## IoT API Endpoints

| Method | Endpoint | Description |
| --- | --- | --- |
| POST | /api/alert | Arduino posts sensor data (JSON) |
| POST | /api/alert/image | ESP32-CAM uploads captured JPEG image |
| POST | /api/heartbeat | Device keep-alive ping (every 5 min) |
| POST | /api/alert/simulate | Admin-only test alert injection |
| GET | /api/alerts/poll | Dashboard JS polls every 5s for new alerts |
| POST | /api/alerts/read | Mark alert(s) as read |
| POST | /api/alerts/dismiss | Dismiss an alert |

## Dashboard Features

Real-Time Notification Bell — navbar badge updates every 5 seconds via JavaScript polling

Notification Drawer — slide-in panel listing recent alerts with level, location, time

Alert Modal Popup — full-screen modal for DANGER/CRITICAL events with audio alarm

Camera Image Display — captured flood/accident photos shown alongside alert details

Alert Log Page — full history with filters by level, dismiss, and Google Maps links

Dashboard Widget — unread alert count and 5 most recent alerts on main dashboard

Simulate Alert Button — admin can inject test alerts without hardware for demos

# Technology Stack

## Hardware & IoT

Microcontrollers: Arduino Uno (ATmega328P) + AI-Thinker ESP32-CAM

WiFi: ESP32-CAM built-in 2.4GHz WiFi (no separate shield)

Camera: OV2640 (built into ESP32-CAM board)

GSM: SIM900A module for SMS alerts

Sensors: Resistive water level (analog), Rain sensor with LM393 (digital), SW-420 vibration (digital)

LEDs: 3x 5mm (Green, Yellow, Red) with 220Ω resistors

Power: LM2596 buck converter (5V + 4V outputs)

Programming: C++ (Arduino Core)

## Web Application

Backend: PHP 7.4+ with custom lightweight MVC architecture

Database: MySQL / MariaDB via PDO with prepared statements

Frontend: HTML5, CSS3, Bootstrap 5, FontAwesome 6, Vanilla JS (ES6+)

Typography: Google Fonts (IBM Plex Sans & IBM Plex Mono)

Audio Alerts: Web Audio API (synthetic alarm tones, no MP3 files needed)

Server: XAMPP (Apache + MySQL)

Version Control: Git (GitHub repository)

SafeSense — IoT-Based Rain and Flood Hazard Monitoring

Arduino Uno + ESP32-CAM + SIM900A GSM + PHP Dashboard